RESPONSE PACKET (QCC -> RC) - FULL 90-BYTE BIT-LEVEL TABLE

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Destination ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | QCC fills this by swapping - echoes the command's Source ID back as this response's Destination ID. |
| 2 | Source ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | QCC fills this by swapping - echoes the command's Destination ID back as this response's Source ID. |
| 3 | Packet Size - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Total packet size = 2970 bytes (90-byte header + 2880-byte QTRM data block). Same fixed value in both the command and the response - QCC does not compute a different size, it writes the same constant. |
| 4 | Packet Size - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 5 | Echo Byte | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Echoes byte 5 from the command packet unchanged. QCC no longer interprets this byte. |
| 6 | Command/Acknowledgement | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x01 | QCC fills 0x01, indicating this packet is QCC's response back to RC. |
| 7 | Message Number - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Counter value echoed back from the command, so RC can identify which message this response corresponds to. |
| 8 | Message Number | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 9 | Message Number | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 10 | Message Number - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 11 | DATE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-31 (dec) | Copied unchanged from the command by QCC. Decimal value (not hex), 01-31. |
| 12 | MONTH | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-12 (dec) | Copied unchanged from the command by QCC. Decimal value (not hex), 01-12. |
| 13 | YEAR - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | XX (dec) | Copied unchanged from the command by QCC. Current year, decimal (not hex). Lower Order Byte and Higher Order Byte combine per byte order (little-endian) to form the full decimal year value. |
| 14 | YEAR - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | XX (dec) |  |
| 15 | Time of the day - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Copied exactly from the command by QCC, unchanged. Exact format/units still TBD. |
| 16 | Time of the day | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 17 | Time of the day | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 18 | Time of the day - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 19 | QCC Query Count - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Response-only: count of messages received by QCC (queries). u32 little-endian. |
| 20 | QCC Query Count | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX | Response-only: QCC Query Count middle byte 2. |
| 21 | QCC Query Count | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX | Response-only: QCC Query Count middle byte 3. |
| 22 | QCC Query Count - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX | Response-only: QCC Query Count higher byte 4. |
| 23 | QCC Response Count - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Response-only: count of messages QCC has responded to. u32 little-endian. |
| 24 | QCC Response Count | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX | Response-only: QCC Response Count middle byte 2. |
| 25 | QCC Response Count | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX | Response-only: QCC Response Count middle byte 3. |
| 26 | QCC Response Count - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX | Response-only: QCC Response Count higher byte 4. |
| 27 | APPLICATION_FIRMWARE_VERSION | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Response-only: application firmware version/build number, hardcoded in the QCC C application code. |
| 28 | RTL_FIRMWARE_VERSION | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Response-only: RTL firmware version/build number, read from a PIO register. |
| 29 | Reserved-1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Reserved Byte | Reserved Byte |
| 30 | Reserved-2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Reserved Byte | Reserved Byte |
| 31 | Reserved-3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Reserved Byte | Reserved Byte |
| 32 | Reserved-4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte (previously a standalone checksum - removed; single CRC-8 at byte 90 now covers the whole header) |
| 33 | QCC_COMMAND (Echo) | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Echoes the QCC_COMMAND value (byte 33) from the command that produced this response, identifying which of the 9 commands was executed. |
| 34 | FPGA_TEMPERATURE - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | FPGA die temperature. 10 usable bits, 2's complement signed value in bits 0-9; bits 10-15 = 0 (fixed). |
| 35 | FPGA_TEMPERATURE - Higher Order Byte | 0 | 0 | 0 | 0 | 0 | 0 | b09 | b08 | 0xXX |  |
| 36 | BOARD_TEMPERATURE - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Board temperature sensor reading, 16-bit. |
| 37 | BOARD_TEMPERATURE - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 38 | BOARD_HUMIDITY - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Board humidity sensor reading, 16-bit. |
| 39 | BOARD_HUMIDITY - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 40 | INPUT_SOB_COUNT - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | SOB count measured on input. |
| 41 | INPUT_SOB_COUNT | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 42 | INPUT_SOB_COUNT | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 43 | INPUT_SOB_COUNT - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 44 | INPUT_PRT_COUNT - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | PRT count measured on input. |
| 45 | INPUT_PRT_COUNT | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 46 | INPUT_PRT_COUNT | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 47 | INPUT_PRT_COUNT - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 48 | INPUT_PPS_COUNT - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | PPS count measured on input. |
| 49 | INPUT_PPS_COUNT | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 50 | INPUT_PPS_COUNT | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 51 | INPUT_PPS_COUNT - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 52 | OUTPUT_PRT_COUNT - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | PRT count measured on output. |
| 53 | OUTPUT_PRT_COUNT | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 54 | OUTPUT_PRT_COUNT | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 55 | OUTPUT_PRT_COUNT - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 56 | OUTPUT_SOB_COUNT - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | SOB count measured on output. |
| 57 | OUTPUT_SOB_COUNT | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 58 | OUTPUT_SOB_COUNT | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 59 | OUTPUT_SOB_COUNT - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 60 | INPUT_SOB_WIDTH_US - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Last SOB pulse width, input, microseconds. |
| 61 | INPUT_SOB_WIDTH_US - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 62 | OUTPUT_SOB_WIDTH_US - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Last SOB pulse width, output, microseconds. |
| 63 | OUTPUT_SOB_WIDTH_US - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 64 | INPUT_PRT_WIDTH_US - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Last PRT pulse width, input, microseconds. |
| 65 | INPUT_PRT_WIDTH_US - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 66 | OUTPUT_PRT_WIDTH_US - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Last PRT pulse width, output, microseconds. |
| 67 | OUTPUT_PRT_WIDTH_US - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 68 | INPUT_PRT_PRI - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | PRT PRI (Pulse Repetition Interval) measured on input, microseconds (32-bit). |
| 69 | INPUT_PRT_PRI | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 70 | INPUT_PRT_PRI | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 71 | INPUT_PRT_PRI - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 72 | OUTPUT_PRT_PRI - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | PRT PRI (Pulse Repetition Interval) measured on output, microseconds (32-bit). |
| 73 | OUTPUT_PRT_PRI | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 74 | OUTPUT_PRT_PRI | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 75 | OUTPUT_PRT_PRI - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 76 | INPUT_PPS_WIDTH_US - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Last PPS pulse width, input, microseconds. |
| 77 | INPUT_PPS_WIDTH_US - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 78 | PPS_COUNTER - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Separate 32-bit PPS counter (distinct from INPUT_PPS_COUNT above). |
| 79 | PPS_COUNTER | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 80 | PPS_COUNTER | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 81 | PPS_COUNTER - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 82 | GENERATOR_STATUS | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Bit 0: SOB_STATE (0=external, 1=internal). Bit 1: PRT_STATE (0=external, 1=internal). Bit 2: QCC_MODE / speed toggle status (0=normal high-speed mode, 1=low-speed remote-programming mode). Bits 7-3: reserved. |
| 83 | DIP_SWITCH | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Hardware DIP switch value. All 8 bits significant - no defined per-bit meaning; GUI shows bits 7-0 individually plus decimal. |
| 84 | Reserved-16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 85 | Reserved-17 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 86 | CHIP_ID - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Lower 32 bits of the 64-bit chip ID, positioned immediately before the Checksum. |
| 87 | CHIP_ID | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 88 | CHIP_ID | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 89 | CHIP_ID - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 90 | Checksum | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | CRC-8/CCITT over bytes 1-89 (poly 0x07, init 0x00, no reflection, xorout 0x00) |

TX PACKET (RC -> QCC) - QCC_COMMAND = 0x00 (DATA_DISTRIBUTION)

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Destination ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Source ID when it builds its response. |
| 2 | Source ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Destination ID when it builds its response. |
| 3 | Packet Size - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Total packet size = 2970 bytes (90-byte header + 2880-byte QTRM data block). Same fixed value in both the command and the response - QCC does not compute a different size, it writes the same constant. |
| 4 | Packet Size - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 5 | Echo Byte | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC may send any value; QCC no longer interprets this byte and echoes it back unchanged in the response (byte 5). |
| 6 | Command/Acknowledgement | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC fills 0x00, indicating this packet is a command sent by RC to QCC. |
| 7 | Message Number - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Counter of messages sent by RC to QCC, incremented per message. |
| 8 | Message Number | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 9 | Message Number | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 10 | Message Number - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 11 | DATE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-31 (dec) | Filled by RC. Decimal value (not hex), 01-31. |
| 12 | MONTH | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-12 (dec) | Filled by RC. Decimal value (not hex), 01-12. |
| 13 | YEAR - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | XX (dec) | Filled by RC. Current year, decimal (not hex). Lower Order Byte and Higher Order Byte combine per byte order (little-endian) to form the full decimal year value. |
| 14 | YEAR - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | XX (dec) |  |
| 15 | Time of the day - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Filled by RC (GUI). Exact format/units (e.g. ms-since-midnight vs packed HHMMSS) still TBD. |
| 16 | Time of the day | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 17 | Time of the day | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 18 | Time of the day - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 19 | Reserved-0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 20 | Reserved-1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 21 | Reserved-2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 22 | Reserved-3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 23 | Reserved-4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 24 | Reserved-5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 25 | Reserved-6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 26 | Reserved-7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 27 | Reserved-8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 28 | Reserved-9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 29 | Reserved-10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 30 | Reserved-11 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 31 | Reserved-12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 32 | Reserved-13 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte (previously a standalone checksum - removed; single CRC-8 at byte 90 now covers the whole header) |
| 33 | QCC_COMMAND | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | Selects DATA_DISTRIBUTION. Triggers a DMA write (mSGDMA); QCC responds with the full 90-byte Response header plus the full, populated 2880-byte QTRM data block. |
| 34-89 | MESSAGE_BODY | - | - | - | - | - | - | - | - | 0x00 |  |
| 90 | Checksum | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | CRC-8/CCITT over bytes 1-89 (poly 0x07, init 0x00, no reflection, xorout 0x00) |

TX PACKET (RC -> QCC) - QCC_COMMAND = 0x01 (QCC_STATUS)

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Destination ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Source ID when it builds its response. |
| 2 | Source ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Destination ID when it builds its response. |
| 3 | Packet Size - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Total packet size = 2970 bytes (90-byte header + 2880-byte QTRM data block). Same fixed value in both the command and the response - QCC does not compute a different size, it writes the same constant. |
| 4 | Packet Size - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 5 | Echo Byte | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC may send any value; QCC no longer interprets this byte and echoes it back unchanged in the response (byte 5). |
| 6 | Command/Acknowledgement | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC fills 0x00, indicating this packet is a command sent by RC to QCC. |
| 7 | Message Number - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Counter of messages sent by RC to QCC, incremented per message. |
| 8 | Message Number | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 9 | Message Number | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 10 | Message Number - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 11 | DATE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-31 (dec) | Filled by RC. Decimal value (not hex), 01-31. |
| 12 | MONTH | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-12 (dec) | Filled by RC. Decimal value (not hex), 01-12. |
| 13 | YEAR - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | XX (dec) | Filled by RC. Current year, decimal (not hex). Lower Order Byte and Higher Order Byte combine per byte order (little-endian) to form the full decimal year value. |
| 14 | YEAR - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | XX (dec) |  |
| 15 | Time of the day - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Filled by RC (GUI). Exact format/units (e.g. ms-since-midnight vs packed HHMMSS) still TBD. |
| 16 | Time of the day | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 17 | Time of the day | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 18 | Time of the day - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 19 | Reserved-0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 20 | Reserved-1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 21 | Reserved-2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 22 | Reserved-3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 23 | Reserved-4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 24 | Reserved-5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 25 | Reserved-6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 26 | Reserved-7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 27 | Reserved-8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 28 | Reserved-9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 29 | Reserved-10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 30 | Reserved-11 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 31 | Reserved-12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 32 | Reserved-13 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte (previously a standalone checksum - removed; single CRC-8 at byte 90 now covers the whole header) |
| 33 | QCC_COMMAND | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x01 | Selects QCC_STATUS. No DMA triggered; QCC responds with current telemetry in the 90-byte Response header only. |
| 34-89 | MESSAGE_BODY | - | - | - | - | - | - | - | - | 0x00 |  |
| 90 | Checksum | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | CRC-8/CCITT over bytes 1-89 (poly 0x07, init 0x00, no reflection, xorout 0x00) |

TX PACKET (RC -> QCC) - QCC_COMMAND = 0x02 (QCC_RESET)

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Destination ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Source ID when it builds its response. |
| 2 | Source ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Destination ID when it builds its response. |
| 3 | Packet Size - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Total packet size = 2970 bytes (90-byte header + 2880-byte QTRM data block). Same fixed value in both the command and the response - QCC does not compute a different size, it writes the same constant. |
| 4 | Packet Size - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 5 | Echo Byte | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC may send any value; QCC no longer interprets this byte and echoes it back unchanged in the response (byte 5). |
| 6 | Command/Acknowledgement | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC fills 0x00, indicating this packet is a command sent by RC to QCC. |
| 7 | Message Number - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Counter of messages sent by RC to QCC, incremented per message. |
| 8 | Message Number | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 9 | Message Number | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 10 | Message Number - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 11 | DATE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-31 (dec) | Filled by RC. Decimal value (not hex), 01-31. |
| 12 | MONTH | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-12 (dec) | Filled by RC. Decimal value (not hex), 01-12. |
| 13 | YEAR - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | XX (dec) | Filled by RC. Current year, decimal (not hex). Lower Order Byte and Higher Order Byte combine per byte order (little-endian) to form the full decimal year value. |
| 14 | YEAR - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | XX (dec) |  |
| 15 | Time of the day - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Filled by RC (GUI). Exact format/units (e.g. ms-since-midnight vs packed HHMMSS) still TBD. |
| 16 | Time of the day | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 17 | Time of the day | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 18 | Time of the day - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 19 | Reserved-0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 20 | Reserved-1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 21 | Reserved-2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 22 | Reserved-3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 23 | Reserved-4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 24 | Reserved-5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 25 | Reserved-6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 26 | Reserved-7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 27 | Reserved-8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 28 | Reserved-9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 29 | Reserved-10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 30 | Reserved-11 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 31 | Reserved-12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 32 | Reserved-13 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte (previously a standalone checksum - removed; single CRC-8 at byte 90 now covers the whole header) |
| 33 | QCC_COMMAND | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x02 | Selects QCC_RESET. No DMA triggered; resets FPGA-side buffers, counters, and internal state via a PIO pin. |
| 34-89 | MESSAGE_BODY | - | - | - | - | - | - | - | - | 0x00 |  |
| 90 | Checksum | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | CRC-8/CCITT over bytes 1-89 (poly 0x07, init 0x00, no reflection, xorout 0x00) |

TX PACKET (RC -> QCC) - QCC_COMMAND = 0x03 (PRT_BYPASS)

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Destination ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Source ID when it builds its response. |
| 2 | Source ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Destination ID when it builds its response. |
| 3 | Packet Size - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Total packet size = 2970 bytes (90-byte header + 2880-byte QTRM data block). Same fixed value in both the command and the response - QCC does not compute a different size, it writes the same constant. |
| 4 | Packet Size - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 5 | Echo Byte | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC may send any value; QCC no longer interprets this byte and echoes it back unchanged in the response (byte 5). |
| 6 | Command/Acknowledgement | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC fills 0x00, indicating this packet is a command sent by RC to QCC. |
| 7 | Message Number - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Counter of messages sent by RC to QCC, incremented per message. |
| 8 | Message Number | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 9 | Message Number | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 10 | Message Number - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 11 | DATE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-31 (dec) | Filled by RC. Decimal value (not hex), 01-31. |
| 12 | MONTH | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-12 (dec) | Filled by RC. Decimal value (not hex), 01-12. |
| 13 | YEAR - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | XX (dec) | Filled by RC. Current year, decimal (not hex). Lower Order Byte and Higher Order Byte combine per byte order (little-endian) to form the full decimal year value. |
| 14 | YEAR - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | XX (dec) |  |
| 15 | Time of the day - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Filled by RC (GUI). Exact format/units (e.g. ms-since-midnight vs packed HHMMSS) still TBD. |
| 16 | Time of the day | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 17 | Time of the day | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 18 | Time of the day - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 19 | Reserved-0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 20 | Reserved-1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 21 | Reserved-2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 22 | Reserved-3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 23 | Reserved-4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 24 | Reserved-5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 25 | Reserved-6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 26 | Reserved-7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 27 | Reserved-8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 28 | Reserved-9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 29 | Reserved-10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 30 | Reserved-11 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 31 | Reserved-12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 32 | Reserved-13 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte (previously a standalone checksum - removed; single CRC-8 at byte 90 now covers the whole header) |
| 33 | QCC_COMMAND | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x03 | Selects PRT_BYPASS. No DMA triggered; PRT generator is bypassed using the externally-supplied parameters below. |
| 34 | PRT_COUNT - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX |  |
| 35 | PRT_COUNT | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 36 | PRT_COUNT | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 37 | PRT_COUNT - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 38 | PRI_WIDTH_US - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX |  |
| 39 | PRI_WIDTH_US | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 40 | PRI_WIDTH_US | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 41 | PRI_WIDTH_US - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 42 | PRT_WIDTH_US - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX |  |
| 43 | PRT_WIDTH_US - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 44-89 | Reserved | - | - | - | - | - | - | - | - | 0x00 |  |
| 90 | Checksum | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | CRC-8/CCITT over bytes 1-89 (poly 0x07, init 0x00, no reflection, xorout 0x00) |

TX PACKET (RC -> QCC) - QCC_COMMAND = 0x04 (SOB_BYPASS)

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Destination ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Source ID when it builds its response. |
| 2 | Source ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Destination ID when it builds its response. |
| 3 | Packet Size - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Total packet size = 2970 bytes (90-byte header + 2880-byte QTRM data block). Same fixed value in both the command and the response - QCC does not compute a different size, it writes the same constant. |
| 4 | Packet Size - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 5 | Echo Byte | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC may send any value; QCC no longer interprets this byte and echoes it back unchanged in the response (byte 5). |
| 6 | Command/Acknowledgement | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC fills 0x00, indicating this packet is a command sent by RC to QCC. |
| 7 | Message Number - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Counter of messages sent by RC to QCC, incremented per message. |
| 8 | Message Number | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 9 | Message Number | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 10 | Message Number - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 11 | DATE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-31 (dec) | Filled by RC. Decimal value (not hex), 01-31. |
| 12 | MONTH | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-12 (dec) | Filled by RC. Decimal value (not hex), 01-12. |
| 13 | YEAR - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | XX (dec) | Filled by RC. Current year, decimal (not hex). Lower Order Byte and Higher Order Byte combine per byte order (little-endian) to form the full decimal year value. |
| 14 | YEAR - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | XX (dec) |  |
| 15 | Time of the day - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Filled by RC (GUI). Exact format/units (e.g. ms-since-midnight vs packed HHMMSS) still TBD. |
| 16 | Time of the day | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 17 | Time of the day | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 18 | Time of the day - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 19 | Reserved-0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 20 | Reserved-1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 21 | Reserved-2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 22 | Reserved-3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 23 | Reserved-4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 24 | Reserved-5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 25 | Reserved-6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 26 | Reserved-7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 27 | Reserved-8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 28 | Reserved-9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 29 | Reserved-10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 30 | Reserved-11 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 31 | Reserved-12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 32 | Reserved-13 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte (previously a standalone checksum - removed; single CRC-8 at byte 90 now covers the whole header) |
| 33 | QCC_COMMAND | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x04 | Selects SOB_BYPASS. No DMA triggered; SOB generator is bypassed using the externally-supplied width below. |
| 34 | SOB_WIDTH - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX |  |
| 35 | SOB_WIDTH - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 36-89 | Reserved | - | - | - | - | - | - | - | - | 0x00 |  |
| 90 | Checksum | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | CRC-8/CCITT over bytes 1-89 (poly 0x07, init 0x00, no reflection, xorout 0x00) |

TX PACKET (RC -> QCC) - QCC_COMMAND = 0x05 (PRT_INTERNAL_GEN)

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Destination ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Source ID when it builds its response. |
| 2 | Source ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Destination ID when it builds its response. |
| 3 | Packet Size - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Total packet size = 2970 bytes (90-byte header + 2880-byte QTRM data block). Same fixed value in both the command and the response - QCC does not compute a different size, it writes the same constant. |
| 4 | Packet Size - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 5 | Echo Byte | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC may send any value; QCC no longer interprets this byte and echoes it back unchanged in the response (byte 5). |
| 6 | Command/Acknowledgement | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC fills 0x00, indicating this packet is a command sent by RC to QCC. |
| 7 | Message Number - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Counter of messages sent by RC to QCC, incremented per message. |
| 8 | Message Number | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 9 | Message Number | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 10 | Message Number - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 11 | DATE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-31 (dec) | Filled by RC. Decimal value (not hex), 01-31. |
| 12 | MONTH | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-12 (dec) | Filled by RC. Decimal value (not hex), 01-12. |
| 13 | YEAR - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | XX (dec) | Filled by RC. Current year, decimal (not hex). Lower Order Byte and Higher Order Byte combine per byte order (little-endian) to form the full decimal year value. |
| 14 | YEAR - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | XX (dec) |  |
| 15 | Time of the day - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Filled by RC (GUI). Exact format/units (e.g. ms-since-midnight vs packed HHMMSS) still TBD. |
| 16 | Time of the day | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 17 | Time of the day | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 18 | Time of the day - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 19 | Reserved-0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 20 | Reserved-1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 21 | Reserved-2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 22 | Reserved-3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 23 | Reserved-4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 24 | Reserved-5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 25 | Reserved-6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 26 | Reserved-7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 27 | Reserved-8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 28 | Reserved-9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 29 | Reserved-10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 30 | Reserved-11 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 31 | Reserved-12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 32 | Reserved-13 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte (previously a standalone checksum - removed; single CRC-8 at byte 90 now covers the whole header) |
| 33 | QCC_COMMAND | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x05 | Selects PRT_INTERNAL_GEN. No DMA triggered; configures the PRT internal generator using the parameters below. |
| 34 | PRT_COUNT - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX |  |
| 35 | PRT_COUNT | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 36 | PRT_COUNT | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 37 | PRT_COUNT - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 38 | PRI_WIDTH_US - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX |  |
| 39 | PRI_WIDTH_US | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 40 | PRI_WIDTH_US | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 41 | PRI_WIDTH_US - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 42 | PRT_WIDTH_US - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX |  |
| 43 | PRT_WIDTH_US - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 44-89 | Reserved | - | - | - | - | - | - | - | - | 0x00 |  |
| 90 | Checksum | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | CRC-8/CCITT over bytes 1-89 (poly 0x07, init 0x00, no reflection, xorout 0x00) |

TX PACKET (RC -> QCC) - QCC_COMMAND = 0x06 (SOB_INTERNAL_GEN)

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Destination ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Source ID when it builds its response. |
| 2 | Source ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Destination ID when it builds its response. |
| 3 | Packet Size - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Total packet size = 2970 bytes (90-byte header + 2880-byte QTRM data block). Same fixed value in both the command and the response - QCC does not compute a different size, it writes the same constant. |
| 4 | Packet Size - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 5 | Echo Byte | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC may send any value; QCC no longer interprets this byte and echoes it back unchanged in the response (byte 5). |
| 6 | Command/Acknowledgement | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC fills 0x00, indicating this packet is a command sent by RC to QCC. |
| 7 | Message Number - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Counter of messages sent by RC to QCC, incremented per message. |
| 8 | Message Number | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 9 | Message Number | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 10 | Message Number - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 11 | DATE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-31 (dec) | Filled by RC. Decimal value (not hex), 01-31. |
| 12 | MONTH | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-12 (dec) | Filled by RC. Decimal value (not hex), 01-12. |
| 13 | YEAR - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | XX (dec) | Filled by RC. Current year, decimal (not hex). Lower Order Byte and Higher Order Byte combine per byte order (little-endian) to form the full decimal year value. |
| 14 | YEAR - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | XX (dec) |  |
| 15 | Time of the day - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Filled by RC (GUI). Exact format/units (e.g. ms-since-midnight vs packed HHMMSS) still TBD. |
| 16 | Time of the day | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 17 | Time of the day | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 18 | Time of the day - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 19 | Reserved-0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 20 | Reserved-1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 21 | Reserved-2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 22 | Reserved-3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 23 | Reserved-4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 24 | Reserved-5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 25 | Reserved-6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 26 | Reserved-7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 27 | Reserved-8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 28 | Reserved-9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 29 | Reserved-10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 30 | Reserved-11 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 31 | Reserved-12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 32 | Reserved-13 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte (previously a standalone checksum - removed; single CRC-8 at byte 90 now covers the whole header) |
| 33 | QCC_COMMAND | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x06 | Selects SOB_INTERNAL_GEN. No DMA triggered; configures the SOB internal generator using the width below. |
| 34 | SOB_WIDTH - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX |  |
| 35 | SOB_WIDTH - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 36-89 | Reserved | - | - | - | - | - | - | - | - | 0x00 |  |
| 90 | Checksum | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | CRC-8/CCITT over bytes 1-89 (poly 0x07, init 0x00, no reflection, xorout 0x00) |

TX PACKET (RC -> QCC) - QCC_COMMAND = 0x07 (PPS_INTERNAL_GEN)

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Destination ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Source ID when it builds its response. |
| 2 | Source ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Destination ID when it builds its response. |
| 3 | Packet Size - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Total packet size = 2970 bytes (90-byte header + 2880-byte QTRM data block). Same fixed value in both the command and the response - QCC does not compute a different size, it writes the same constant. |
| 4 | Packet Size - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 5 | Echo Byte | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC may send any value; QCC no longer interprets this byte and echoes it back unchanged in the response (byte 5). |
| 6 | Command/Acknowledgement | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC fills 0x00, indicating this packet is a command sent by RC to QCC. |
| 7 | Message Number - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Counter of messages sent by RC to QCC, incremented per message. |
| 8 | Message Number | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 9 | Message Number | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 10 | Message Number - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 11 | DATE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-31 (dec) | Filled by RC. Decimal value (not hex), 01-31. |
| 12 | MONTH | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-12 (dec) | Filled by RC. Decimal value (not hex), 01-12. |
| 13 | YEAR - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | XX (dec) | Filled by RC. Current year, decimal (not hex). Lower Order Byte and Higher Order Byte combine per byte order (little-endian) to form the full decimal year value. |
| 14 | YEAR - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | XX (dec) |  |
| 15 | Time of the day - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Filled by RC (GUI). Exact format/units (e.g. ms-since-midnight vs packed HHMMSS) still TBD. |
| 16 | Time of the day | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 17 | Time of the day | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 18 | Time of the day - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 19 | Reserved-0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 20 | Reserved-1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 21 | Reserved-2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 22 | Reserved-3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 23 | Reserved-4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 24 | Reserved-5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 25 | Reserved-6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 26 | Reserved-7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 27 | Reserved-8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 28 | Reserved-9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 29 | Reserved-10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 30 | Reserved-11 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 31 | Reserved-12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 32 | Reserved-13 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte (previously a standalone checksum - removed; single CRC-8 at byte 90 now covers the whole header) |
| 33 | QCC_COMMAND | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x07 | Selects PPS_INTERNAL_GEN. No DMA triggered; configures the PPS internal generator using the width below. |
| 34 | PPS_WIDTH - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX |  |
| 35 | PPS_WIDTH - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 36-89 | Reserved | - | - | - | - | - | - | - | - | 0x00 |  |
| 90 | Checksum | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | CRC-8/CCITT over bytes 1-89 (poly 0x07, init 0x00, no reflection, xorout 0x00) |

TX PACKET (RC -> QCC) - QCC_COMMAND = 0xFF (REMOTE_PROGRAMMING) - 90-BYTE HEADER (shared by every Remote Programming frame variant; trailing bytes for each SubCommand are in the tables below)

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Destination ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Source ID when it builds its response. |
| 2 | Source ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Filled by RC (GUI). QCC will swap this into Destination ID when it builds its response. |
| 3 | Packet Size - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Total packet size = 2970 bytes (90-byte header + 2880-byte QTRM data block). Same fixed value in both the command and the response - QCC does not compute a different size, it writes the same constant. |
| 4 | Packet Size - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 5 | Echo Byte | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC may send any value; QCC no longer interprets this byte and echoes it back unchanged in the response (byte 5). |
| 6 | Command/Acknowledgement | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | RC fills 0x00, indicating this packet is a command sent by RC to QCC. |
| 7 | Message Number - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Counter of messages sent by RC to QCC, incremented per message. |
| 8 | Message Number | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 9 | Message Number | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 10 | Message Number - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 11 | DATE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-31 (dec) | Filled by RC. Decimal value (not hex), 01-31. |
| 12 | MONTH | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 01-12 (dec) | Filled by RC. Decimal value (not hex), 01-12. |
| 13 | YEAR - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | XX (dec) | Filled by RC. Current year, decimal (not hex). Lower Order Byte and Higher Order Byte combine per byte order (little-endian) to form the full decimal year value. |
| 14 | YEAR - Higher Order Byte | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | XX (dec) |  |
| 15 | Time of the day - Lower Order Byte | b07 | b06 | b05 | b04 | b03 | b02 | b01 | b00 | 0xXX | Filled by RC (GUI). Exact format/units (e.g. ms-since-midnight vs packed HHMMSS) still TBD. |
| 16 | Time of the day | b15 | b14 | b13 | b12 | b11 | b10 | b09 | b08 | 0xXX |  |
| 17 | Time of the day | b23 | b22 | b21 | b20 | b19 | b18 | b17 | b16 | 0xXX |  |
| 18 | Time of the day - Higher Order Byte | b31 | b30 | b29 | b28 | b27 | b26 | b25 | b24 | 0xXX |  |
| 19-32 | Reserved | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte(s) |
| 33 | QCC_COMMAND | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xFF | Selects a Remote Programming session. No DMA triggered. Byte 34 (MESSAGE_BODY offset 0) is a SubCommand QCC itself reads and acts on - see byte 34 row below and the per-SubCommand tables that follow this one. |
| 34 | SUB_COMMAND | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00-0x02 | SubCommand QCC itself reads and acts on for this Remote Programming frame. 0x01 = QCC -> Low-Speed (Mode Step 2) and 0x02 = QCC -> High-Speed: QCC also latches byte 35 (QTRM_SELECT) into its LRU-select mux, and the frame ends at byte 90, nothing follows. 0x00 = Broadcast: QCC fans a further command out over the low-speed link - to all 96 QTRMs, or to just the one QTRM latched by the preceding Mode Step 2's QTRM_SELECT - see the SubCommand 0x00 tables below for the exact trailing bytes per QTRM command (Link Check, Get LRU Info, Bitstream Receive, Firmware Update, Bitstream DATA, QTRM -> High Speed, and Mode Step 1's per-QTRM-slot broadcast). |
| 35 | QTRM_SELECT | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00-0x5F / 0xFF | Target QTRM selector, read by QCC only when SUB_COMMAND is 0x01 or 0x02: 0x00-0x5F = single QTRM id (0-based, 0-95), 0xFF = broadcast to all 96 QTRMs. Latched into QCC's remote-programming LRU-select mux for the whole low-speed session, so every subsequent SubCommand 0x00 frame reaches only the selected QTRM (or all 96 if 0xFF), and QCC zero-fills the 95 non-selected QTRM slots in its 2970-byte responses. Not read under SubCommand 0x00 - RC sends 0x00 there. |
| 36-89 | Reserved | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte(s) |
| 90 | Checksum | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | CRC-8/CCITT over bytes 1-89 (poly 0x07, init 0x00, no reflection, xorout 0x00) |

REMOTE_PROGRAMMING, SubCommand = 0x00 (Broadcast) - Inner Command = Link Check (bootloader command_type 0x30) - bytes 91-100

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 91 | INNER_HEADER | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xAA | Bootloader inner-command header byte (BL_HEADER). |
| 92 | INNER_PACKET_SIZE_ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | Packet-size/class identifier - 0x00 for every fixed 10-byte inner command. |
| 93 | INNER_COMMAND_TYPE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x30 | Link Request command type. |
| 94 | STATUS_BYTE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | sub_status_type (bits 7-4) / status_type (bits 3-0). |
| 95-99 | Reserved | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte(s) |
| 100 | INNER_CHECKSUM | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | XOR of the preceding 9 inner-command bytes (PROVISIONAL algorithm - see bootloader_packet.py's bootloader_checksum()). |

REMOTE_PROGRAMMING, SubCommand = 0x00 (Broadcast) - Inner Command = Get LRU Info (bootloader command_type 0x31) - bytes 91-100

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 91 | INNER_HEADER | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xAA | Bootloader inner-command header byte (BL_HEADER). |
| 92 | INNER_PACKET_SIZE_ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | Packet-size/class identifier - 0x00 for every fixed 10-byte inner command. |
| 93 | INNER_COMMAND_TYPE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x31 | Get LRU Info request command type (request-side layout ASSUMED - the IDD only defines 0x31 as the LRU->RC response; see bootloader_packet.py open item 10). |
| 94 | STATUS_BYTE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | sub_status_type (bits 7-4) / status_type (bits 3-0). |
| 95-99 | Reserved | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte(s) |
| 100 | INNER_CHECKSUM | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | XOR of the preceding 9 inner-command bytes (PROVISIONAL algorithm - see bootloader_packet.py's bootloader_checksum()). |

REMOTE_PROGRAMMING, SubCommand = 0x00 (Broadcast) - Inner Command = Bitstream Receive announce (bootloader command_type 0x33, post-MSS) - bytes 91-100

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 91 | INNER_HEADER | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xAA | Bootloader inner-command header byte (BL_HEADER). |
| 92 | INNER_PACKET_SIZE_ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | Packet-size/class identifier - 0x00 for every fixed 10-byte inner command. |
| 93 | INNER_COMMAND_TYPE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x33 | Bitstream Receive command type - shares 0x33 with the Mode Change command; QTRM-state-dependent overload (pre-MSS = Mode Change, post-MSS = Bitstream Receive), never ambiguous at runtime. |
| 94 | IMAGE_IS_GOLDEN | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00/0x01 | 0 = Current image, 1 = Golden image. |
| 95 | Reserved | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 96 | FW_PACKET_SIZE_LSB | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Bitstream chunk size in bytes, little-endian - lower byte. Fixed 4096 (RP_PAYLOAD_SIZE). |
| 97 | FW_PACKET_SIZE_MSB | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Bitstream chunk size in bytes, little-endian - upper byte. |
| 98 | FW_PACKET_COUNT_LSB | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Total number of bitstream chunks in this transfer, little-endian - lower byte. |
| 99 | FW_PACKET_COUNT_MSB | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Total number of bitstream chunks in this transfer, little-endian - upper byte. |
| 100 | INNER_CHECKSUM | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | XOR of the preceding 9 inner-command bytes (PROVISIONAL algorithm - see bootloader_packet.py's bootloader_checksum()). |

REMOTE_PROGRAMMING, SubCommand = 0x00 (Broadcast) - Inner Command = Firmware Update Command / Authenticate, Verify, Program (bootloader command_type 0x36) - bytes 91-100

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 91 | INNER_HEADER | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xAA | Bootloader inner-command header byte (BL_HEADER). |
| 92 | INNER_PACKET_SIZE_ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | Packet-size/class identifier - 0x00 for every fixed 10-byte inner command. |
| 93 | INNER_COMMAND_TYPE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x36 | Firmware Update Command REQUEST command type (own opcode, does not share 0x35). |
| 94 | IMAGE_IS_GOLDEN | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00/0x01 | 0 = Current image, 1 = Golden image. |
| 95 | Reserved | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 96 | IAP_MODE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x01/0x02/0x03 | 1 = AUTHENTICATE, 2 = PROGRAM, 3 = VERIFY. |
| 97-99 | Reserved | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte(s) |
| 100 | INNER_CHECKSUM | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | XOR of the preceding 9 inner-command bytes (PROVISIONAL algorithm - see bootloader_packet.py's bootloader_checksum()). |

REMOTE_PROGRAMMING, SubCommand = 0x00 (Broadcast) - Inner Command = QTRM -> High Speed / Mode Change MSS->Fabric (bootloader command_type 0x32) - bytes 91-100

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 91 | INNER_HEADER | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xAA | Bootloader inner-command header byte (BL_HEADER). |
| 92 | INNER_PACKET_SIZE_ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | Packet-size/class identifier - 0x00 for every fixed 10-byte inner command. |
| 93 | INNER_COMMAND_TYPE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x32 | Mode Change MSS->Fabric command type - tells the QTRM processor to hand the UART back to the fabric (return to high speed). Firmware handler only toggles GPIOs and exits; no reply is expected. |
| 94-99 | Reserved | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte(s) |
| 100 | INNER_CHECKSUM | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | XOR of the preceding 9 inner-command bytes (PROVISIONAL algorithm - see bootloader_packet.py's bootloader_checksum()). |

REMOTE_PROGRAMMING, SubCommand = 0x00 (Broadcast) - Inner Command = Bitstream DATA chunk (bootloader command_type 0x34) - bytes 91-4196

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 91 | INNER_HEADER | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xAA | Bootloader inner-command header byte (BL_HEADER). |
| 92 | INNER_PACKET_SIZE_ID | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xBB | PSI_BITSTREAM - distinct from the 0x00 fixed-size marker, since this is the one variable-length inner command. |
| 93 | INNER_COMMAND_TYPE | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x34 | Bitstream Data Packet command type. |
| 94 | Reserved | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte |
| 95 | FW_PACKET_LENGTH_LSB | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Real length of this chunk in bytes, little-endian - lower byte (the final short chunk keeps its real length here even though DATA below is 0xFF-padded to 4096). |
| 96 | FW_PACKET_LENGTH_MSB | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Real length of this chunk in bytes, little-endian - upper byte. |
| 97 | FW_PACKET_COUNT_LSB | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Total number of bitstream chunks in this transfer, little-endian - lower byte. |
| 98 | FW_PACKET_COUNT_MSB | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Total number of bitstream chunks in this transfer, little-endian - upper byte. |
| 99 | FW_PACKET_INDEX_LSB | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Zero-based index of this chunk, little-endian - lower byte. |
| 100 | FW_PACKET_INDEX_MSB | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | Zero-based index of this chunk, little-endian - upper byte. NOT a checksum byte, unlike every other inner command - this command has no trailing checksum (open item: unconfirmed whether one exists for this packet type). |
| 101-4196 | DATA | - | - | - | - | - | - | - | - | 0xXX... | The actual SPI bitstream chunk payload, up to 4096 bytes (RP_PAYLOAD_SIZE) - opaque application data with no further byte-level fields, so shown as one range rather than one row per byte. The final chunk is 0xFF-padded to the full 4096 bytes; FW_PACKET_LENGTH above carries its real (unpadded) length. |

REMOTE_PROGRAMMING, SubCommand = 0x00 (Broadcast), Mode Step 1 ONLY (QTRMs -> Low-Speed) - Inner Command = Mode Change (bootloader command_type 0x33, pre-MSS), replicated into all 96 x 30-byte QTRM slots - bytes 91-2970

| Byte No. | Field Name | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | Value in Hex | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 91 | INNER_HEADER (Slot 1) | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xAA | Bootloader inner-command header byte (BL_HEADER). Mode Step 1 is the one Remote Programming frame that is NOT [90-byte header][10-byte command] - QTRMs are still per-QTRM-addressed at this point (not yet on the shared low-speed FIFO), so this same 10-byte command is replicated into the first 10 bytes of each of the 96 x 30-byte QTRM slots that make up the standard 2970-byte frame. |
| 92 | INNER_PACKET_SIZE_ID (Slot 1) | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00 | Packet-size/class identifier - 0x00 for every fixed 10-byte inner command. |
| 93 | INNER_COMMAND_TYPE (Slot 1) | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x33 | Mode Change command type - shares 0x33 with Bitstream Receive; QTRM-state-dependent overload (pre-MSS = Mode Change, decoded by the fabric RTL; post-MSS = Bitstream Receive, decoded by the processor). |
| 94-95 | Reserved (Slot 1) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte(s) |
| 96 | BSN_MODE (Slot 1) | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0x00-0x03 | Bits B04-B01 (bits 7-4 reserved). 0=INITIALISATION, 1=OPERATION, 2=MAINTENANCE, 3=MSS_CONTROL (the value Mode Step 1 actually sends - "switch to the low-speed 115200 bootloader link"). |
| 97-99 | Reserved (Slot 1) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Reserved Byte(s) |
| 100 | INNER_CHECKSUM (Slot 1) | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | 0xXX | XOR of the preceding 9 inner-command bytes (PROVISIONAL algorithm - see bootloader_packet.py's bootloader_checksum()). |
| 101-120 | Reserved (Slot 1, bytes 11-30) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0x00 | Unused - only the first 10 bytes of each 30-byte QTRM slot carry the Mode Change command; the remaining 20 bytes per slot are zero-padded. |
| 121-2970 | QTRM Slots 2-96 | - | - | - | - | - | - | - | - | (repeats) | Each of the remaining 95 QTRM slots repeats the exact same 30-byte structure as Slot 1 above (bytes 91-120). Broadcast session (QTRM_SELECT 0xFF): the identical 10-byte Mode Change command in every slot. Single-QTRM session: only the target QTRM's slot carries the command - the other 95 slots are all-zero, so only that QTRM drops to low-speed. |